# Calvert Industrial Mutual Aid Program

## Calvert City, Kentucky

# Member Company Emergency Alarm Description

| Company | Type(s) Of alarm | Test Schedule | Emergency Alarm Description | All Clear Alarm Description |
| --- | --- | --- | --- | --- |

| Evonik Wacker Sekisui | 4 electrically powered stationary alarms with solar powered battery backup systems – wireless activation from Main Gatehouse (CIMAP Base Station) | Every Friday @ 1200 (noon) | Active Shooter – Command 5 Alarm Weather Emergency – Solid Wail Alarm Fire – Hi-Lo Alarm General Emergency – Oscillating Air Horn | Westminster Chimes |
| --- | --- | --- | --- | --- |
| Arkema | Outside alarms w/verbal instruction capability. Indoor alarms w/verbal instruction, Hallway speakers, Telephones w/emergency alerting ring system and computer link to exact location of emergency | Daily 0700 and 1200, Phone/computer emergency system test every Friday at 1200 | Bell tone followed by verbal direction to indicate what action to take | Melody of “The Entertainer” |
| Ashland Chemicals | Two omni directional outdoor speaker towers, Multiple remote indoor speakers for elevated noise locations, Strobe lights demarking assembly points | 1300 hrs on the first Wednesday of each month | General Emergency - "wailing" tone (15 second tone...two audible pre-recorded messages indicating the location of emergency...15 second tone) Production Area / Plant Evacuation - alternating "high/low" tone (30 second duration) | "Buzzer" tone (30 second duration) |
| Evonik Degussa Corporation | Outside Alarms. Strobe lights activated in high noise areas. Intercom capability. | Second Wednesday of each Month @ 1400 | Steady Horn – Shelter In Place Wailing Tone – Plant Evacuation | Westminster Chime |

# Calvert Industrial Mutual Aid Program

## Calvert City, Kentucky

# Member Company Emergency Alarm Description

| Company | Type(s) Of alarm | Test Schedule | Emergency Alarm Description | All Clear Alarm Description |
| --- | --- | --- | --- | --- |

| Estron Chemical, Inc | Emergency Siren | First Wednesday of each Month @ 10:00 AM | Single Tone, Solid Wail | Verbal |
| --- | --- | --- | --- | --- |
| Westlake Chemical Corporation Lubrizol | Outdoor Air Horn | Friday @ 11:45 AM | Coded Number of Blast | Two (2) Single Blast |
| Westlake PVC Corporation | Two Outdoor Omni Directional Speaker Towers | Friday @ 1:00 PM | General Emergency- "Pulse Steady" tone. Followed by audible pre-recorded voice messages indicating the location of emergency. (Repeated two times) Plant Evacuation – “Alternate Steady" tone. Followed by audible pre-recorded voice messages | “Steady Tone” Followed by audible pre-recorded “ALL Clear” voice message. |

# Marshall County Warning Siren

| County Outdoor Warning Siren | Severe Weather Emergency Alarm | First Friday of Each Month @ 1:00 PM | Solid Wail | N/A |
| --- | --- | --- | --- | --- |

2021 Outdoor Warning Siren Test Schedule

_____________________________________________________________

Date                                                     Initiator                                           Type of Test

January 1                                       MC 911/CIMAP                                  C2 (Haz-Mat)

February 5                                             MC 911                                         C2 (Haz-Mat)

March 5			      MC 911			   C1 (Weather)

April 2                                              MC 911/CIMAP                                C1 (Weather)

May 7                                                    MC 911			   C2 (Haz-Mat)

June 4                                                   MC 911			   C2 (Haz-Mat)

July 2  		                     MC 911/CIMAP		                  C1 (Weather)

August 6			    MC 911			   C1 (Weather)

September 3			   MC 911			  C2 (Haz-Mat)

October 1                                        MC 911/CIMAP 		                 C2 (Haz-Mat)

November 5                                           MC 911                                          C1 (Weather)

December 3                                           MC 911                                          C1 (Weather)

- Each month MC 911 will initiate the County wide sirens ...

…( including the Calvert Area)

- On every 3rd month CIMAP Base will initiate the Calvert Area Siren

- The dates listed are the FIRST FRIDAY of each month and the

test will be conducted at 1 PM.

- The dates listed will also be the schedule for the School Radio

test conducted at 7:30 AM

- Will try to send a reminder the Wednesday before.